Memo Lab 3

To: Dr. Hisham Sager

From: Norlen Li and Luke Boyd, Group 11

Date: March 11, 2026

Problem Statement

The goal of this lab is to read the values of the encoders and battery voltage of the Atmega16u4 using the external pin changes. This would keep track of both the wheel rotations to determine position and orientation as well as keeping track of the battery voltage to make sure that the car does not have dead batteries.

Methods

To complete Lab 3 there were 3 main parts, the battery monitor code and system, the encoder code and system, and the serial communication and support code.

The battery monitors mainly relied on the ADCSRA register read analog signals of the battery voltage [1]. Where the ADSC pin of the ADCSRA register was read to find the analog signal. To initialize the ADCSRA register, a prescaler of 128 is set to transform the 16MHz clock to 125kHz. This is achieved by setting bits ADPS0, ADPS1, and ADPS2 to 1 according to Table 24-5 [1]. Setting REFS0 and REFS1 in register ADMUX to 1 created the digital voltage reference of 2.56. Setting MUX0, MUX1, and MUX3 on register ADMUX and MUX5 on ADCSRB to 1 sets the analog input for the positive differential input to ADC7 and the negative differential input to ADC0 with a gain of 10. Finally setting bit ADEN in register ADCSRA to 1 enables the analog to digital converter.

To read the battery voltage the ADSC bit of ADCSRA register is set to 1 and the analog signal is read. Once the value is read, ADSC is set back to 0. This value, however, was read from 0-1023 and had to be scaled back to a voltage reading. To find this voltage scaler ‘BITS_TO_BATTERY_VOLTS’ equation 1 was used. Where vref is 2.56.

ADC=2vin⋅1023vref#1	Multiplying the value read by BITS_TO_BATTERY_VOLTS gives the final voltage of the batteries. Even though the battery voltage has been read, it must still be filtered to eliminate any noise from the battery readings. The filter used was a first order impulse response filter (IIR) low pass filter. This allows the filtering of high pass noise from the battery while the slow decline is allowed through to show the true battery voltage. The filter design uses a first order filter, shown by equation 2 as it only depends on the current and previous voltage values.

voltage=A⋅voltageread-B⋅voltageprev#2

Because there are two encoders on the robot, twice as many registers and bits need to be initialized. To initialize the registers to read the encoder values, first, register EIMSK bit INT6 is set to 0 to prevent accidental triggers of INT6 before proper initialization. For the right encoder, EICRB bit ISC61 is set to 0 and ISC60 is set to 1, this makes it so that any logical change on INT6 generates an interruption request, shown in section 11.1.2 [1]. Similarly for the left encoder, PCICR bit PCIE0 and PCMSK0 bit PCINT4 are set to 1. This interrupt setting is important because any time the encoders change state, they create an interrupt so that the counts can be read. Next, the data direction registers DDRB bit 4, DDRE bits 2 and 6, and DDRF bit 0 to 0. This allows the reading of the left XOR, left encoder channel B, right XOR and right channel B to be input. The pullup resistors must be enabled to prevent floating inputs. This is done by setting register PORTB bit PB4, PORTE bits PE2 and PE6, and PORTF bit PF0 to 1. Finally, EIFR is set to 0 to create a clean slate after initialization and EIMSK bit INT6 is set to 0 to reenable the interrupt after being disabled at the beginning of initialization.

After initialization the encoder values are automatically updated using the interrupt service routines (ISRs). There is one for each encoder, and each is triggered by any change of the channel or XOR pins. The current state of channel B and the reconstructed state of channel A (A=B ^ XOR) are read. When comparing these states to the previous states of channel A and B the ISR can determine whether to increment or decrement the count value of the encoder. The count values are declared as volatile integers so they can be safely accessed by both the main functions and ISRs. When the main functions read the count variables the interrupts are temporarily disabled to ensure safe reading. These counts are also transformed into radian values for easier manipulation by other functions. Knowing that there are 12 counts of the encoder for each revolution, equation 3 can be used to make the transform.

Encoderrad=2π⋅Encodercounts12#3

Unlike the battery monitor, the encoder counts do not need to be filtered because the encoders produce discrete digital signals that are less susceptible to noise. The encoder also reads counts that need to be accumulated over time which are not susceptible to small analog noise like the battery voltage is.

Once the battery monitor and encoder code has been created, the supplementary systems need to be edited or created. The Filter.c code was to create the IIR filter to filter the battery voltage. Message_Handling.c was edited to include commands for ‘b’ to read the battery voltage once, ‘B’ to read the battery voltage every specified time in ms, ‘e’ to read the encoder counts once, ‘E’ to read the encoder counts every specified time in ms. The Lab3_Tasks.c and .h files needed to be written. Lab3_Tasks.c includes all the functions to call the battery monitor and encoder functions. This includes sending battery voltage and encoder values once and, in a loop, filtering the battery voltage and sending a ‘BAT LOW’ message if the battery voltage is below 4 volts. The Lab3_Tasks.h header file creates all of the tasks for each of these actions. Finally, the Lab3-Encoder.c file was written. This file initializes all of the support functions for message handling, task handling, serial communication etc. The file also initializes all the tasks created to check the encoder values and battery voltage. Lab3-Encoder.c also includes the main loop which runs the code that allows tasks to be run.

Results

Future Work

For the future, using this code can allow us to accurately control where the robot moves as it drives. It allows the use of a differential drive control scheme to control where the robot translates and rotates accurately. The battery code acts more to help the user and does not have much future improvements.

Appendix

Works Cited

[1] Atmel-7766JS-USB-ATmega16U4/32U4-datasheet_04/2016 features, https://ww1.microchip.com/downloads/en/DeviceDoc/Atmel-7766-8-bit-AVR-ATmega16U4-32U4_Summary.pdf (accessed Feb. 21, 2026).